UNIVERSITY OF TORONTO

Department of Electrical and Computer Engineering

Fields and Waves Laboratory

Courses ECE 320F and ECE 357S

III Year

RESONANT CAVITY

Network Analyzer

- Introduction

Network analyzer is a combination of a sweep generator and receiver which can measure either reflected or transmitted signals.  The received signals can be displayed in various formats such as SWR and impedance magnitude.

The user can control the source parameters such as frequency and sweep width.

The operation of the receiver can be controlled as to the format of display and scaling.

- Operation of Network Analyzer

is controlled by:

- Function selection buttons (3, 4, 5, 6, 8 on the attached drawing) and,

- mode selection keys (9 on the drawing) prompted by CRT display.

- Operating Procedure

- Switch on.

- System options button (6).

Preset button (8).

Basic receiver control.

- Measure 1 button (3).

- Reflection key (9).

- Frequency button (4).

Center frequency key (9)

Sweep generator control.

- 550 MHz numeric keys (7)

- Span key (9)

- 200 MHz numeric keys (7)

- Format button (8)

Smith chart key (9)

Display control.

- Marker button (5)

- Move marker using dial to real positive reflection point on the Smith chart.  Read marker frequency.

- Frequency button (4)

Center frequency key (9).

Sweep generator adjustment.

- Adjust to frequency in step 14 above.

- Span key (9).

- 50 MHz numeric keys (7).

- Format button (5).

- SWR key (9).

Scale button (5).

Display control.

- Autoscale key (9).

- Marker button (5).

- Move marker to minimum of SWR and read frequency.

Frequency button (5).

Sweep generator control.

- Center frequency key (9)

- Adjust frequency to step 25 above.

Format button (5).

Display control.

- More format key (9).

- Impedance magnitude key (9).

Phase offset key (9).

Choice of reflection coefficient position.

- Adjust phase offset until peak of impedance coincides with marker of step 25 above.